Tire Size

Notes

- Make sure to enable data logging and connect/backup to Google Drive

- 2018 Trucks need security bypass cable

- Tire size is stored in the BCM

- Tire size must match in BCM and ABS settings

- Run ABS Initialization in ABS diagnostic after size change

Procedure

Key in RUN position

Open App

Connect to OBD II device

Select DODGE RAM 1500 or 2500 or 3500

From drop down menu select BODY COMPUTER

- MY2013-17 (2018)

- CONNECT

Click on the CAR WITH THE OPEN HOOD

From the drop down menu select PROGRAM TIRE SIZE

  - From here you can either choose from a selected list of tire sizes or enter your own in MM  (circumference)

Press START

Once complete, back out to the main page

Cycle the key (OFF wait 1 MIN, RUN)

Now your instrument cluster should be lit up.

Go back into CAR CONFIGURATION CHANGE > Select VEHCONFIG 4- DYNAMIC TIRE CIRCUMFERENCE (ALL OR REAR)

Enter value and select START

ABS MODULE RESET

Select DODGE RAM 1500 or 2500 or 3500

From drop down menu select ABS

- ABS ATE/CONTINENTAL

Click on the CAR WITH THE HOOD OPEN

From the drop down list select STATIC ECU INITIALIZATION                                 Click on CHECK ENGINE icon

>Read the instructions and hit START                                                                                        > CLEAR FAULTS

CYCLE KEY (OFF, RUN)

Click CAR ICON

From drop down menu select CLEARING ROLLS INFORMATION

- START

Clear faults in BODY CONTROL MODULE > Clear faults in ENGINE

CYCLE KEY (OFF, RUN)

EXIT program and check speed with GPS

Note

You May have to select ENGINE > Select MY CHRYSLER PENISTAR 3.6/4.7/5.7/6.4 HEMI> Select the CHECK ENGINE Icon and clear any codes